Documentation for the TMS320C30-Part of the cc68k C-Compiler

============================================================

1. Introduction

---------------

In this Documentation are the TMS320C30 specific parts of the

cc68k Compiler explained. There are also some hints about

interfacing C-code and assembler-code. The cc68k compiler

may produce code for 680xx, 80x86, 8086 and TMS320c30.

The compilervariant producing TMS320C30-Code is called

cc30 and referenced as this in the rest of this document.

Foe general Informations and Options of the cc68k, see the

cc68k Documentation, wich should be included with this

distribution. If not, see .....

There is also an explanation included how to build the

different or a combination of the needed compilervariants.

The produced assemblersource can be assembled with the as30_2.4

package from Ted Rossin if it is modified to accept simple expressions).

Sinc Ted Rossins assembler should be compatible with the official TI

Assembler the TI-assembler should accept the generated asm-source too,

but I had no possibility to try it. If there should be some problems I

will correct them as soon as I get informations abaut them.

25. Sept. 1996, Ivo Oesch, Selzweg 1, 3422 Kirchberg, Switzerland.

email: b19oesch@isbe.ch (valid til march 1997)

2. History of this document

---------------------------

Created: 20.09.1996 by Ivo Oesch

Status:  Still under construction

History: 25.09.1996 first prerelease

05.11.1996 added trapp-infos

Actual compilerversion:

4.5 beta 25 Oct 96

3. Contents

-----------

1.  Introduction

2.  History of this document

3.  Contents

4.  Quick start

5.  Options of the cc30

6.  What to do if the compiled code does not work

7.  Known Bugs

8.  Work to do

9.  Used COFF-Sections

10. Interfacing to assemblercode

11. Special trapp-handling

12. Restrictions

13. peephole-optimizer

14. What's new (currently nothing)

15. History of the TMS320C30-Codegenerator

4. Quick start

-------------

To start the compiler, simply type

cc30 -asm=yes -g -v <source.p> <source.asm>

(assumed You called the Compilerexecutable cc30)

source.p   Preprocessed inputsourcefile, which must not contain

any other preprocessordirectives than #pragma or #line,

since in cc30 there is no preprocessor included.

If not present, cc30 reads from stdin.

source.asm Outputfile to write assembleroutput, if not present

cc30 writes to stdout.

-asm=yes   means, that the nonstandardkeyword asm allowing

the including off assemblerstatements in the C-source

is enabled.

Example  asm(" LDI 0,ST ");

-g         Puts Source-linenumbers into the assembleroutput,

and if it was enabled at compiletime, also the

whole c-sourcelines are put into the assembleroutput

Makes debugging somewhat easier.

-v         Verbose, puts some messages abaut compilerprogress

into stderr. (Normaly not needed)

For more details about cc30, resp cc68, see the Doc for the cc68k

Compilers

5. Options of the cc30

----------------------

The options  listed in this section will only be available

if support  for the Texas Instruments TMS320C30 DSP processor

was specified at the time the compiler was built.

-collect=yes|no

Enable remove of redundant moves, does only

supress the harder searchs for redundants.

This option may be removed soon or combined

with others.

(Don't ask me where this optins name comes from)

-delayed=n

n is a number between 0 and 3 and is defining

the conditions under wich delayed branches are

used:

0: No delayed branches are used

1: At least one usefull instruction must follow

for using delayed branches (or not more than

2 nops are allowed to be included in order

to use the delayed branch

2: At least two usefull instruction must follow

for using delayed branches (or not more than

1 nop is allowed to be included in order

to use the delayed branch

3: All three instructions following the delayed

must be usefull for using delayed branches

(or no nop is allowed to be included in order

to use the delayed branch

The usefull instructions are Taken from bevore

the branch (The branchinstruction is moved

backwards) and if this is not sufficient, also

instructions from the Branchtarget are copied

behind the branch and the branch-target (label)

is moved acordingly.

-mul32=yes|no

If enabled, real 32Bit*32Bit multiplication is

used for longs. Otherwise the TMS320C30 24Bit*

24Bit multiplication is used also for longs.

for int's and char's the 24Bit version is allways

used.

-optpipeline=yes|no

Enables peephole-optimization to minimize pipe-

line conflicts resulting from usage of address-

registers as operands in instructions and for

addressgeneration. It is trying to rearange code-

sequences to minimnize those pipeline-conflicts

-parallel=yes|no

Enables usage of parallel instructions if

possible. This is done in the peephole-optimizer

wich analyzes codesequences and replaces

instructions with their parallel aequivalence

in the moment only parallel ldi||ldi, ldi||sti

and sti||sti are suported

-probe=no|yes

specify whether stack probes should be generated

when each stack frame is built.   This option is

only  available   if  the   PROBES  option   was

configuration option  was set  when the compiler

was built.

Default:  -probe=no

-pseq=string

mainly for peepoptimizer-finetuning, the string

is in the form nnn/n...n/nnn/..../n, where n is

a number between 0 and 6.

Valid strings: 6

54321/6/54321

54/3/2/1

Meaning all each numbergroup describes wich optins

are done in this pass, and there are as much

optimizer passes as numbergroups are.

The numbers means:

0: Do only simple standard-optimizations

1: Combine fwd, arithmetic instr with a following

load is combined to a 3-op instuction if possible

2: Combine bwd, arithmetic instr with a preceeding

load is combined to a 3-op instuction if possible

3: Commutative, if a commutative arithmetc/logical

instruction is followed by a load, and the load

can be avoided if the operands are swapped, this

is done and the load deleted.

4: Do Remap, description see remap-option below

5: Redundant, description see option collect

6: Dataflow, description see option optdataflow

The current internal aequivalence of this option used

in the compiler is 54321/6/543210.

You may experiment a little with it and have a look

at teh results. If You find a better sequence, let

me know

-ramconst=yes|no

Enables putting big integerconstants (more than

16 bits) into the .const segment wich must be in

the same datapage as all other datasegments

(.bss and .data)

Otherwise integerconstants bigger than 16bits are

constructed with ldi, shift and or. (I don't know

if this option is really usefull, it may be if

You cant spare any bit of space in the datapage.

-remap=yes|no

Enables remaping of registers, e.g an ldi rx,ry is

removed if ry can be replaced with rx in the

following code, or if rx can be replaced with ry

in the preceding codesequence.

This option uses abaut 50% more of compiletime if

enabled.

-shortfloat=n

Determines in wich cases the shortfloatconstantes

are used.

n may be 0,1 or 2:

0 shortfloatconstants are only used if we are

absolutely sure they wont bring any loss in

precision in the given constant

1 Constants of the type float are allways represented

in shortform if they are in range, double and

longdouble only if there is no loss in precision

2 float-, double- and longdouble-constants are

allways represented in the shortform if they are

in range (between -255 and +255), there may be a

loss in precision

-interrupt=yes|no

If set to yes, in each function there ara allways

all used registers saved onto the stack.

(In the moment allways all Registers are saved,

there is no check if they are also used. That will

change in future, of course

This Option is thougth to be used in the form

#pragma interrupt=yes

Interruptroutine

#pragma interrupt=no

If it is set true in the commandline, all functions

inside this module are allways saving all registers.

-forcedsave=option_list

Forces the compiler to save all listed registers on

functionsentry and restore them when leaving a

function. Makes usually only sense for special

interruptrutines, should not be switched on in the

comandline.

This Option is thougth to be used in the form

#pragma forcedsave=all

special Interruptroutine

#pragma forcedsave=none

option_list may be any combination of registernames,

also valid are 'all', 'rn' 'arn' for all registers

resp. for all dataregisters or all addressregisters

example for valid options:

...=r0, r1, rc, re, arn

...=all

...=rn, dp, ar6

...=none

If added this option sinc I needed it to get an

realtime-operatinsystem running, was needed for

context-switching.

-optbranch=none|low|medium|hard

Deceides the amount of branchoptimizations done in

the peepholeoptimizer.

none:    Nothing is done

low:     only moving of blocks is allowed and

replacing conditional jumps over loads with

conditional loads

medium:  additional to the above if the code before

the branch instruction is the same as the

instructions before the label the branch is

moved back.

hard:    additional to the above Codeblocks with same

instructionssequence ending with jumps to same

label are commoned up

-optdataflow=yes|no

Enables Dataflowanalyzer. If enabled, the optimizer

walks through the code and keeps track of the

contents of the registers and replace each data-

acces with the cheapest way, if possible.

Warning, this optimization has a high probability

to violate a volatiole-constraint.

Workaround:

If You want use this optimization and have a

volatile-access, put a dummy-asm-statement like

asm("*Dummy, stops dataflowanalyzer") before this

statement, then the dataflower loses control over

dataflow in this part and can't do any replacements

this option needs abaout 16% more compiletime if

enabled.

-opt3operand=yes|no

If enabled, the optimizer converts whenever possible

two-operand-instructions into three-operand

instructions. This may open new paths for the other

optimizer-stages.

If support  for multiple  processors and/or assemblers was

specified when  the  compiler  was  built,  then  you  can

specify the  target to  be a  TI TMS320C30  processor  and  a

specific assembler  can be  specified using  the following

options:

-rosc30   Generate code for the TI TMS320C30 processor in the

Rossin assembler format (Should be compatible with

the official TI-Assemblerformat).

For more informations about the work of the peepholeoptimizer

and how some of the options above may influence it, see the

chapter 'peephole-optimizer'.

6. What to do if the compiled code does not work

------------------------------------------------

If Your code seems not doing what it should, try first the option

-peep=no, that disables the whole peephole-optimizer. If then

everitning works ok, some opzimization displays bad behaviour.

otherwise its the Codegenerator, then nothing can be done, except

inform me about the bug.

Otherwise You may now try to disable spezific Optimizations using

the options explained abbove.

7. Known Bugs

-------------

There is no Way to generate Interruptroutines (reti and saving of

all registers), but that should be fixed in the next version.

Peepholeoptimizer may violate volatile constraint, especially if

the dataflowanalyzer is at work. Can be fixed in the moment

with an asm-statement like asm("*Stop Peepholeoptimizer");

before the Volatile-access. (The peepholeoptimizer cannot work

over asm-statements, because he knows nothing abaut what they do.

if You get a message like

"FATAL error encountered in file '%s' routine '%s'\nMessage: "

"\n(this may occur due to a feature left out of the compiler, or just an

\ninternal compiler error)")

You have found an internal incosistency. If You are using the -v option

Then the last function printed out did generate this error.

Please send me the code of this function and the exact message

wich was printed out, and then I have somthing to do...

(Normall singlestep trough the code to find out whats gone wrong)

8. Work to do

-------------

-Not imlemented now:

-More usage of the conditional loads. (instead of conditional

branches around a load).

-Remove uneeded stores to the locals at the end of a function

-Make an option for a large Memorymodel, so there are no restrictions

abaut a 64k-page for statics, strings and globals. But it will cost

an extra load of the dp-register for every acces to such an variable

so for me that hasn't high priority.

-Try to find a better method to evaluate indexnodes.

9. Used COFF-Sections

---------------------

Sections used by this compiler:

This Compiler compiles in folowing sections:

.bss   Uninitialized datas (Statics and globals)

.rom   Strings (if You are compiling in the K&R-mode [-trad=yes]

strings are placed into the .data-segment. ATTENTION, in

the standard-mode, duplicate strings are generated only

once, so don't try to modify them during programflow. thats

also the reason to put them into the .rom-segment

.text  Code and jumptables for switches

.data  Initialized datas (Statics and globals)

.const Constants (Integers and float > 16bit; Adresses of strings;

AAdresses of jumptables for switch)

,bss, .data and .const MUST be located in the same datapage, sinc the

dp-register is only initialized at startup and must then stay constant

So there may not be more than 64Kword in those segments together.

The .const and .data must be initialized during startup or downloading

the code (Sinc I'm using downloading, there is no initializingroutine

included in the library).

If You want to put the code into an ROM, You must must link the code

so that:

.rom   has load and runaddress in the ROM.

.text  has load and runaddress in the ROM.

.bss   has load and runaddress in the DATA_RAM

.data  has load address in the ROM and

runaddres in the DATA_RAM.

.const has load address in the ROM and

runaddres in the DATA_RAM.

If You are using a ROM, You must copy the .data and the .const

segments from their load-location(in the ROM) to their RUN-

location (in the RAM) before You are calling _main.

RAM_DATA is the 64Kpage in which all Variables and constants of

the compiler must be located, and to which the DP-Register must

be set before calling _main.

SP must point to a reasonable amount of free RAM anywhere in

memory, for fast local-varable-access, onchip-ram is recomended.

Warning, if You are using recursion, deep functionsnesting are

a lot of local (automatic) varibles, the space in the internal

Ram may not be sufficient.

Example for an C-Strtuproutine, there is missing the copyloop

to copy the initialized datas and constsegments into the ram.

There is also no support for argc and argv, and the returnvalue

is simply ignored.

(If You need argv and argc, simply push them onto stack before

calling _main, first push th pointer argv, then push the value

argc, and the returnvalue of main will be delivered in r0)

.global    C_start

.global    _main

.global    _exit

.global    _InitMemMgt

.data

C_Stack  .word Stack           ;  Stack grows to higher addresses

.bss       old_stack, 1

.usect "Stack", 2048  ; we need some space for the Stack

.sect "CLIB"          ; Startupcode we want in the section

; CLIB, so we can force the linker

; to place it where we want it.

C_start  push  dp

ldp   .data, dp       ; set data pointer to external RAM

sti   sp, @old_stack  ; save old stackpointer

ldi   @C_Stack, sp    ; initialize Stack

call  _main           ; Calls main from C-Code

_exit                          ; entrypoint for exit function

ldi   @old_stack, sp  ; restore old stackpointer

pop   dp              ; restore old dp

trapu 26              ; breakpoint for monitor

retsu                 ; return to caller

In the library you find some routines needed by the compiler like

startupcode and runtimesupport for divisions. You may change the

contents of this library as you need it.)

10. Interfacing to assemblercode

-------------------------------

10.1 Exchanging variables

------------------------

The Compiler holds initialized static variables and constants wich

are too big for immediates in the .data, resp. .const segment.

If You want to access a c-variable from an assemblermodule, dp must

point to the datapage of the c-code, then You can access this

variable with its name, but You must add an underscore before

the c-name, like

in C-Module

int MyVar;

in assemblermodule

ldi _MyVar, R0

If You want to export an assembleraddress into C-Code, You must

do that in an indirect way:

(the direct way is only possible for locations inside the

64K datapage to wich the register dp of the c-module is pointing.)

in Assemblermodule

.sect"anysect"

MyAsmVar           .word 13

.sect"data"

_AddressOfMyAsmVar .word MyAsmVar

in C-Module

extern int* AddressOfMyAsmVar;

/* Change contents of MyAsmVar */

*AddressOfMyAsmVar = 113;

This is also the way to access Big arrays of memory, which should

not be placed into the 64K datapage (neither in .bss, nor in .data)

in Assemblermodule

MyBigArrayDatas    .usect"BigArray" 50000

.sect"data"

_MyBigArray        .word MyBigArrayDatas

in C-Module

extern int MyBigArray[50000];

/* Change contents of MyBigArray */

MyBigArray[12345] = 113;

10.2 Interfacing Procedures

--------------------------

If you want to call assemblerroutines out of C-functions You must

respect following restrictions:

- A called assemblerroutine must preserve the registers AR2-AR7,

R3-R7 (Float and integer) and dp. The other registers may be used

freely.

- If You want to use locals or acces arguments You must build a

stackframe. C uses AR7 as framepointer, but You are free to use

any other Auxiliaryregister in Your assemblercode.

- A (facultative) returnvalue must be delivered in r0.

Now an example for an C-callable assemblerfunction

* Routine for demonstration of a C-Interface

* no useful function else

*

_MyAsmFunction:

push  AR7          ;Save old Framepointer

ldi   SP,AR7       ;Load Framepointer with new Frameaddr.

addi  3, SP        ;We need space for 3 locals

push  AR4          ;We must save all Used AR-Registers > AR1

push  R4           ;We must save all Used R-Registers > R2

push  R5           ;We must save whole mantissa (32 bits)

pushf R5           ;and exponent + 24bit Mantisse if we

;need a Floatregister

;it is important to save first the whole

;mantissa

* Now we are ready to do some work

ldi   *-AR7(2), R4  ;load first parameter  in R4

;(in our example this is the value 1)

ldi   *-AR7(3), AR4 ;load second parameter in AR4

;(in our example this is the value 2)

ldf   *-AR7(4), R0  ;get third parameter, R0 may be freely used

;(in our example this is the value 3.14)

float R4, R1        ;R1 may be freely used (no push needed)

stf   R1, *+AR7(1)  ;Save result in first local space

addf  1.2, R1       ;

stf   R1, *+AR7(2)  ;save result in second local space

mpyf3 R4,R0,R1      ;

addf  *+AR7(2),R1   ;Add from second local to R1

ldi   13, AR0       ;AR0 may be freely used

sti   AR0, *+AR7(3) ;Only to use third local space

ldf   *+AR7(1), R0  ;R0 is return-value

ldf   0.0, R5       ;only to destroy contents of R5

;so we had a reason to save it

* We have now finished our really useful assemblerroutine

* we have to claenup the stack and restore the registers

* before we return to our caller, result must be in R0

popf  R5            ;restore Exponent and 24bit mantissa

pop   R5            ;restore 32bit mantissa

pop   R4            ;restore R4

pop   AR4           ;restore AR4

ldi   SP, AR7       ;remove  Stackframe

pop   AR7           ;restore framepointer

retsu               ;return to caller

*End of our little ASM-Demoprogram

And now a little C_programm to use our assemblerroutine

/* Prototype for our External assemblerroutine */

extern float MyAsmFunction(int p1, int p2, float p3);

/*Now a main to use our function */

int main(int argc, char *argv[])

{

/* we need some place to store result of our function */

float a;

/*  Call of our fantastic asm-example

*  First  Para = 1 (integer)

*  Second Para = 2 (integer)

*  Third  Para = 3.14 (float)

*/

a = MyAsmFunction(1, 2, 3.14);

return (a+1);

}

If You want to call a C-Routine from assembler, You must push

the arguments (if any) in reverse order onto the stack and then

call the function. If You need the contents of any Scratchregister

(r0,r1,r2,ar0,ar1,ir0,ir1,rc,re,rs, You must save them onto the

Stack before pushing the arguments and calling the C-Function.

The returnvalue of this function will be found in r0. After calling

the function You must clean the stack (Remove teh pushed arguments

of the stack), usually with an subi n,sp instruction.

Example:

int MyLittleFox(int a, float b, float *c)

{

return ((int)(a+b+*c));

}

...

push  r0               ;Save R0, we need it later

ldi   _PtrToFloat, r0  ;Get pointer to Float

push  r0               ;Push as argument 'c'

ldf   3.14, r0         ;Get a float

pushf r0               ;Push as argument 'b'

ldi   17,r0            ;Get an integer

push  r0               ;Push as argument 'a'

call  _MyLittleFox     ;Call our C-Function

ldi   r0,r2            ;get returnvalue

subi  3,sp             ;remove the 3 arguments from stack

pop   r0               ;restore saved r0

addi  r0,r2            ;add to returnvalue

....

WARNING, it is not recomended to pass structures as arguments or

returnvalues, use pointers to the structures instead. (Structures

are handled somewhat special in the compiler, they get some extraspace

on the stackframe, and pointers to them are passed as arguments. But

this may change in future, so use pointers.

11. Special trapp-handling

--------------------------

If a function with a name like __trap_nn, 00<=nn<=31 is called, then

the assemblerinstruction trapu nn is generated, instead of a

functionscall. This is thougt to support Operatingsystem-

calls, if needed in this way.

Parameters of this function are pushed onto the stack as usually,

an the stack is also corrected accordingly after the execution of

the trapp-instruction.

Example:

/* Prototype (of course) */

int __trap_02(int a, int b);

/* Call traproutine */

c=__trap_02(2,6);

hint: traps can be hidden with defines, see example

/* in Header */

int __trap_00(int Command, int Parameter, ...);

#define OsSignal(SemaphoreId) __trap_00(OS_SIGNAL, SemaphoreId)

/* in Code */

OsSignal(MySema);

12. Restrictions

----------------

-In the moment, all statics, globals and strings have to be in the

same 64k-Page and the dp register must point to this page when main

is called.

-It is not recommended to use more than 255 locals and more than

255 arguments in functionscalls, sinc indirect addressing allows

only offsets from -255 to +255. Otherwise the code will get pretty

inefficiently (the address must be calculated by hand)

-If assemblerroutines are called from C, they must preserve the

registers R3 - R7, AR2 - AR7 and dp, AR7 is used as framepointer.

R0-R2, AR0, AR1 and all specialregisters except dp may be used

freely. A returnvalue must be in R0, and arguments can be accessed

throug building of a stackframe (see Interface to Assembler).

But I recomend to save all registers exept r0,r1,r2,ar0,ar1. So You

wont run in incompatibilities if the compiler will use in a later

version some of the other registers also for his own use. The

registers r0, r1, r2, ar0, ar1 will allways be scratchpads for the

compiler and are free to use for any subroutine, as every routine

knows it must save this registers during a functionscall if their

contents is still needed.

-All integer-datatypes have the same size (32 bits).

(Not truly a restriction, only a speciality :-) )

-Multiply uses the TMS mpy operation, so only 24bits*24bits is supported

for longs exists a option for true 32bit multiplication, but its

using more instructions for a simple multiply. The code for the

multiplication is inlined.

-All floatingpoint-datatypes have the same size and precision (24 bit

mantissa and 8 bit exponent)

13. peephole-optimizer

----------------------

Following is a short description of the optimizations performed

by the peepholeoptimizer.

- deletes branches to the following statement

add1 1,r2     becomes addi 1,r2

bxx Lnear

Lnear  ldi  2,r0     Lnear   ldi  2,r0

- Unreferenced labels are removed

- sequences of labels are commoned up

- optimizes conditional branch over a bra.

check if no more than 3 ldi/ldf follows immediately

behind this branch

if the destination is behind this ldi/ldf we may

replace the branch with conditional loads

tstb r1,   r1     becomes   tstb   r1,   r1

bne  Lxxx

ldi  *ar2, r1               ldieq  *ar2, r1

ldi  3,    ar0              ldieq  3,    ar0

ldf  2.0,  r2               ldfeq  2.0,  r2

Lxxx addi r1,   ar0         Lxxx addi   r1,   ar0

- Space optimisation:

if the code before the target of the branch is itself a branch

then we can move the destination block of code to eliminate the

branch

ldi 1,r0    becomes       ldi 1,r0

br  Lxyz             Lyz  ldi r0,r2

.                         br  Luvw

.                         .

.                         .

br qrs                    .

br  qrs

Lxyz  ldi r0,r2

br  Luvw

- Space optimisation:

if the code before the branch instruction is the same as the

instruction before the label the branch can be moved back an

instruction.

ldi 1,r0    becomes       ldi 1,r0

ld1 r0,r6                 br  Lnew

br  Lxyz                  .

.                         .

.                         .

.                         .

.

ldi r0,r6          Lnew   ldi r0,r6

Lxyz  ldi r0,r2          Lxyz   ldi r0,r2

br  Luvw                  br  Luvw

- Space optimisation:

Optimise for the situation where two branches to the same

target label have identical instruction sequences

leading up to the branch.   We can instead eliminate one

of these instruction sequences by branching to the other one.

ldi  1,r0    becomes      ldi  1,r0

ld1  r0,r6                br   Lnew

addi r3,r0                .

br  Lxyz                  .

.                         .

ldi  2,r0                 ldi  2,r0

ld1  r0,r6         Lnew   ldi  r0,r6

addi r3,r0                addi r3,r0

br  Lxyz                  br   Lxyz

.                         .

.                         .

- Speed optimisation:

if the code before the branch contains no labels and no branches or

calls we use delayed branches (the next three instructions after

the branch are executed before the branch really occurs, therfore

the next three instructiuns cannot be any kind of branches, labels,

asm, calls and jumps

if the branch cannot be moved for three locations, we must fill

in nops

this replacement should only be done in the last pass of the optimizer

e.g when all unneeded labels are eliminated and all codemoves are done

do replace only if at least (opt_delayed_branches) number

of valid instructions will follow in the next 3 instructions

after the branch

(or not more as (3-opt_delayed_branches) nops must be included

this allows some balancing between speed and codesize

case 1 Move branch 3 Instr back

brd  Lxyz

ldi  2,r0      becomes    ldi  2,r0

ld1  r0,r6                ldi  r0,r6

addi r3,r0                addi r3,r0

br  Lxyz                  .

.                         .

.                         .

case 2 Cannot move branch 3 instr back, insert nop

bxx  AnyWhere

bxx  Anywhere             brd  Lxyz

ldi  2,r0      becomes    ldi  2,r0

addi r3,r0                addi r3,r0

br  Lxyz                  nop

.                         .

.                         .

Lxyz  call _Hello         Lxyz  call _Hello

case 3 Cannot move branch 3 instr back, get instr from branch

target and change branchdestination (Lnew)

bxx  AnyWhere

bxx  Anywhere             brd  Lnew

ldi  2,r0      becomes    ldi  2,r0

addi r3,r0                addi r3,r0

br  Lxyz                  push r0

.                         .

.

Lxyz  push  r0            Lxyz  push r0

call _Hello         Lnew  call _Hello

- peephole optimization for stf instructions.

replace following sequence with their parallel aequivalent:

ldf    *ARx, Rm

stf    Rn, *ARy   Rm != Rn

ARx != ARy if both are modified (++/--)

or

stf    Rn, *ARy

stf    Rm, *ARx   ARx != ARy if both are modified (++/--)

*ARxy may be:

*ARxy *+ARxy(1)   *-ARxy(1)   *++ARxy(1)   *--ARxy(1)

*+ARxy(IRn) *-ARxy(IRn) *++ARxy(IRn) *--ARxy(IRn)

- peephole optimization for ldf instructions.

replace following sequence with their parallel aequivalent:

stf    Rn, *ARy

ldf    *ARx, Rm   ARx != ARy if both are modified (++/--)

Above optimization can only be used if we are

shure that ARx and ARy don't point to the same

address, and since this if difficult to tell we don't

otpimize this case

(The only case we can be shure is, when ar and ary are the

same register, but have different offsets)

or

ldf    Rn, *ARy

ldf    Rm, *ARx   ARx != ARy if both are modified (++/--)

*ARxy may be:

*ARxy *+ARxy(1)   *-ARxy(1)   *++ARxy(1)   *--ARxy(1)

*+ARxy(IRn) *-ARxy(IRn) *++ARxy(IRn) *--ARxy(IRn)

- peephole optimization for sti instructions.

replace following sequence with their parallel aequivalent:

ldi    *ARx, Rm

sti    Rn, *ARy   Rm != Rn

ARx != ARy if both are modified (++/--)

or

sti    Rn, *ARy

sti    Rm, *ARx   ARx != ARy if both are modified (++/--)

*ARxy may be:

*ARxy *+ARxy(1)   *-ARxy(1)   *++ARxy(1)   *--ARxy(1)

*+ARxy(IRn) *-ARxy(IRn) *++ARxy(IRn) *--ARxy(IRn)

- peephole optimization for ldi instructions.

replace following sequence with their parallel aequivalent:

sti    Rn, *ARy

ldi    *ARx, Rm   ARx != ARy if both are modified (++/--)

Above optimization can only be used if we are

shure that ARx and ARy don't point to the same

address, and since this if difficult to tell we don't

otpimize this case

(The only case we can be shure is, when ar and ary are the

same register, but have different offsets)

or

ldi    Rn, *ARy

ldi    Rm, *ARx   ARx != ARy if both are modified (++/--)

*ARxy may be:

*ARxy *+ARxy(1)   *-ARxy(1)   *++ARxy(1)   *--ARxy(1)

*+ARxy(IRn) *-ARxy(IRn) *++ARxy(IRn) *--ARxy(IRn)

- peephole optimization for remapping of registers.

assumed we are on a ldi/ldf instruction

if a sequence like this is found beforehead:

ldi   X,rx (dest overwritten) Overwritten marks end of sequence

addi  Y,rx (dest modified)

ldi  rx,Q  (used)

mpyi  Z,rx (dest modified)

ldi  rx,ry         <-ip <<<<<<<<<<<

.

or if a sequence like this is found after ldi/ldf:

ldi  ry,rx         <-ip <<<<<<<<<<<

addi Y, rx (dest modified)

mpyi Z, rx (dest modified)

ldi  rx,Q  (used)

ldi  G, rx (dest overwritten) Overwritten marks end of sequence

.

and in the following statements ry is not used

we delete the ldi instruction and replace rx with

ry in this sequence

- peephole optimization for ldi/ldf

if destination of ldi is never used again and

source does not modify any ar (am_ainc/am_adec)

we can delete this instruction

- peephole optimization for ldi/ldf

Determine whether a move is redundent ... this is done by looking back

along the code list (following all branches to labels) to determine

whether the destination already contains the necessary result.

if so, we can delete the ldi/ldf

- peephole optimization for ldi instructions.

remove ldi rx,rx (they may set the flags but flags are ignored after ldi

eliminate redundant move with registers on sequence:

ldi    Xn, Xm

ldi    Xm, Xn

eliminate redundant move with registers on sequence:

sti    Xn, Xm

ldi    Xm, Xn

remove transfer via memory on sequence:

change

sti    Rx, Xm

ldi    Xm, Ry

to

sti    Rx, Xm

ldi    Rx, Ry

eliminate redundant move with registers on sequence:

or     Xn, Xm  <- Commutative instruction

ldi    Xm, Xn

Xm is not used further

replace with

or     Xm,Xn

combine move with preceding instructipon on sequence:

addi   Xn, Xm   binary instruction

ldi    Xm, Xy

Xm is not used further

replace with

addi3  Xm,Xn,Xy

Combine load followeded by binary instruction to

3-op instruction:

ldi    Xn, Xm

addi   Xr, Xm

Xn, Xr may not by Immediate, Direct

or indirect with offset > +/-1

replace with

addi3  Xm,Xr,Xm

- peephole optimization for ldf instructions.

remove ldf rx,rx (they may set the flags but flags are ignored after ldi

eliminate redundant move with registers on sequence:

ldf    Xn, Xm

ldf    Xm, Xn

eliminate redundant move with registers on sequence:

stf    Xn, Xm

ldf    Xm, Xn

remove transfer via memory on sequence:

change

stf    Rx, Xm

ldf    Xm, Ry

to

stf    Rx, Xm

ldf    Rx, Ry

eliminate redundant move with registers on sequence:

addf   Xn, Xm  <- Commutative instruction

ldf    Xm, Xn

Xm is not used further

replace with

addf     Xm,Xn

combine move with preceding instructipon on sequence:

addf   Xn, Xm   binary instruction

ldf    Xm, Xy

Xm is not used further

replace with

addf3  Xm,Xn,Xy

Combine load followeded by binary instruction to

3-op instruction:

ldf    Xn, Xm

addf   Xr, Xm

Xn, Xr may not by Immediate, Direct

or indirect with offset > +/-1

replace with

addf3  Xm,Xr,Xm

- peephole optimization for sti instructions.

eliminate redundant move with registers on sequence:

ldi    Xn, Xm

sti    Xm, Xn

- peephole optimization for stf instructions.

eliminate redundant move with registers on sequence:

ldf    Xn, Xm

stf    Xm, Xn

- peephole optimization for unconditional transfers. deletes instructions

which have no path. applies to br, bu, and retsu instructions.

(If after a branch or retsu code follows without a leading label

it is unreachable un can be deleted therefore)

- if a label is followed by a branch to another label, the

branch statement can be deleted when the label is moved

- peephole optimization for compare instructions.

changes compare #0,rx to tstb rx

cmpi 0,rn =>    tstb3 rn,rn

but only if following cond. instr is not

using the cary-flag in any way

cary is only used for unsigned compares,

but an unsigned compare against 0

can only be same, not same, or higher

lower is always false, lower-same

is identical to equal, higher is

identical to notequal and

higher same is allways true.

those cases should be simplified best

in the global optimizer

so we simply dont any changes here in

the code if such a case happens

- deletes a tst instruction if the flags are already set.

we assume that op_tst is allways used in the form

tstb rn, rn

that also both operands are the same registers

only operations with dst = R0-R7 can set flags

(except cmp and tst, of course)

- deletes a cmpf instruction if the flags are already set.

aplies only for cmpf 0.0,Rn

- reduce pipelineconflicts:

due write or reads from addressregister

followed be using adressregisters for addressgeneration

ldi    Rz, Rg

add    Rn, Ry

ldi    Xn, Am

ldi   *Ax, Xn <-- Pipelineconflict, 2 cycles delay

can be solved:

ldi    Xn, Am <-- Moved 2 Instr. forward, see (*)

ldi    Rz, Rg <-- Ar cannot be used for Addressgeneration

add    Rn, Ry <-- Ar cannot be used for Addressgeneration

ldi   *Ax, Xn <-- No Pipelineconflict

(*) can only be done if instructionssequence moved is not

depending from the 2 instructions in the middle.

ldi    Xn, Am

ldi   *Ax, Xn <-- Pipelineconflict, 2 cycles delay

ldi    Rz, Rg

add    Rn, Ry

can be solved:

ldi    Xn, Am

ldi    Rz, Rg <-- Ar cannot be used for Addressgeneration

add    Rn, Ry <-- Ar cannot be used for Addressgeneration

ldi   *Ax, Xn <-- Moved 2 Instr. forward, see (*)

No Pipelineconflict now

(*) can only be done if instructionssequence moved is not

depending from the 2 instructions in the middle.

And finally I added an other new optimization, I called it 'dataflow-

analyzer'.

Well it is just following the code and keeping track, wich registers

are containing what for datas (it even follows jumps and loops, it

is doing it so in multiple passes to find out the maximal common set

at points where two or more flows are merging).

It is using this information to take the cheapest copy of the needed

operand, usually dataregisters are preferred before addressregisters

to avoid pipelineconflicts, Registers are preferred bevore immediates

wich allows some other optimizations to produce shorter code, and

registers are preferred before indirect or direc memoryaccess.

The last neeeds some considerations, it may violate an volatile-

constraint, so I think this must be switched of if any volatile is

used, could also be an compileroption...

An example:

this:                 becomes:

ldi *ar2, r0              ldi *ar2, r0

addi  r0, r2              addi  r0, r2

sti   r2, *ar3            sti   r2, *ar3

ldi  *ar2, r3             ldi  *ar2, r3

addi *ar3, r4             addi   r2, r4

addi *ar2, r4             addi   r3, r4

14. What's new (currently nothing)

----------------------------------

Empty (everthing is new :-))

15. History of TMS codegenerator

----------------------------

18. Dec 1995 Started to write an codegenerator for the TMS320C30.

10. Jan 1996 Codegenerator is now finished with some restrictions

(see Work to do) but has still to be tested.

Peephole-optimizer works now for the simplest

optimisations.

3. March 1996 Made some Minor errorcorrections and modifications.

Added delayed branches in peephole optimizer.

20. Sept 1996 Minor errorcorections, enhancements, added a lot more

of peepholeoptimizations, is now nearly complete

(I have no more ideas what to optimize for the moment)

5.  Nov 1996  Some clean-up, removing of unneeded code, added trapp-

support.